OpenFly Telemetry ©
Inertial navigation board

OFT001 v1

©2019 Jonas Bjurel

Licenses:
The official OpenFly Telemetry project(s) are all open source. The licensing may vary from GPL 2, GPL 3, Apache, or Creative Commons. As with most hardware projects – such as OFT001 v1 comes with the Creative Commons - CC BY 2.0 license.

Disclaimers:
The use of any HW, SW or instructions from the OpenFly Telemetry open source project, or any references to other art - is at your own risk. No warranties, or legal obligations shall be assumed from the creators – in any way. Flying model planes, drones or other remotely controlled vehicles always impose danger to the surroundings – make all necessary precautions to make sure that no one is in danger even in case the control of your model is lost for whatever reason.

OpenFly Telemetry overview

OpenFly Telemetry is an open source project aiming to provide telemetry and navigation capabilities for model airplanes and drones through open source hardware and software. It uses various on-board sensor capabilities such as GPS, Accelerometers, Gyroscopes, Magnetometers, Pitot tubes, temperature sensors, battery voltage and current sensors, RPM sensors, etc. to provide the operator or algorithms with data for manual or automated corrective actions. The algorithms are some time autonomous and resides in the model, are sometimes on ground with the receiver or nearby connected devices or are sometimes even in distant clouds.

The OFT modules can also be placed in the module base of the transmitter to enrich the transmitter awareness, e.g. Position, direction and additional communication capabilities such as Bluetooth and WIFI.

Figure 1: OpenFly Telemetry basic idea

OpenFly Telemetry (OFT) system description

OFT has two basic components: the Ada fruit ESP32 Huzzah feather processor board and the OFT system board. The ESP32 processor board obviously holds the software and algorithms for telemetry and navigation. The system board interfaces with the radio receiver and it’s telemetry and servo busses. The system is modular, and you can compose your choice of capabilities by stacking various OFT boards on top of each other.

Figure 2: OpenFly Telemetry system view

OFT is based out of the Ada fruit ESP32 Huzzah feather processor board. This board hosts a ESP32 processor with two 32 bit general cores, 2 float processors and one ultra-low-power co-processor from Esperessif (https://www.espressif.com/). Furthermore, the ESP32 board provides wireless connectivity through Bluetooth and WIFI as well as ensuring continuous power supply through an optional 2S designated LIPO, which feeds the servos and the receiver- should the main supply drain.

The ESP32 mother board connects to several telemetry acquisition boards in a mother-daughter board/shield configurations.

Figure 3: Ada fruit ESP32 Huzzah feather board

For the on-board use cases (as opposed to the TX bay use case), the bottom most shield in the stack holds the OFT System board. The system board has an FPGA that connects to the radio receiver through the S.PORT for telemetry data and the SBUS for servo output. The FPGA will allow the processor to mix in servo corrections from the processor, transmitting upstream telemetry data, as well as receiving upstream telemetry and servo information from the transmitter.

The system board interconnects with the radio control receiver through its S.PORT and SBUS.

Figure 4: Frsky XR4 2.4 GHz MIMO CDMA receiver with S.PORT- (telemetry) and SBUS (Servos) connections

The OFT001 v1 Inertial navigation board

The OFT001 v1 Inertial navigation board captures navigational data, such as GPS positioning, 3D acceleration data, 3D Gyroscopic data, 3D Magnetic data, Barometric pressure, Temperature and Humidity. These data are provided to the ESP32 processor card (and any other shield cards) through I2C and RS232. These data will be filtered, processed and transferred by the ESP32 card and the OFT System card FPGA. It is still to be seen if we can/will implement a high-performance Inertial navigation fuse fixed point Kalman filter in the FPGA, or if it is better done in SW?

Figure 5: OFT001 v1 Inertial navigation card overview

The OFT001 v1 card has a L80-M39 GPS with a built-in patch antenna and an optional connector for an external 50 Ohm active antenna; an MP-9250 3D axis Accelerometer, Gyroscope and Magnetometer; and a BMP280 Barometric pressure- humidity- and temperature sensor. The communication with these sensors have been segmented so that the MPU runs on it’s own high speed I2C channel allowing fast updates and possibly from a the OFT System card FPGA, the BMP280 on a shared I2C channel with many, while the GPS communication happens over an RS232 serial channel using the NMEA protocol.

There is one green LED indicator for GPS lock, one orange LED indictor for External GPS antenna use, and an RGB LED for generic system status indication driven from the ESP32 CPU board. There are also two push buttons: one which is connected to the overall system reset, and another multi-purpose push button intended for system calibration activities.

It is important that OFT001 v1 sits on the top of the mother-daughter board stack (unless an external GPS antenna is used) to achieve a good GPS signal.

Cupper layers have been kept out from ESP32 Bluetooth and WIFI PCB antennas to not deteriorate signal strength.

Figure 5: OFT001 v1 3D simulation – Front to the right

Figure 6: OFT001 v1 3D simulation – Front to the upper left

Figure 7: OFT001 v1 3D simulation – Back view

OFT001 v1 pin-out

| OFT001 12 pin header (J2) | ESP32 HUUZZAH 12 pin header (JP3) | ESP32 silicon | Comments |
| --- | --- | --- | --- |
| 1 - N/C | VBAT |  |  |
| 2 - N/C | EN |  |  |
| 3 - N/C | VBUS |  |  |
| 4 – RED LED | IO13_A12 |  | Red system status led |
| 5 – BLUE LED | IO12_A11 |  | Blue system status led |
| 6 – GREEN LED | IO27_A10 |  | Green system status led |
| 7 - N/C | IO33_A9 |  |  |
| 8 - N/C | IO15_A8 |  |  |
| 9 - N/C | IO32_A7 |  |  |
| 10 - N/C | I14_A6 |  |  |
| 11 - I2C_0_SCL_3V | SCL |  | Common non-critical I2C |
| 12 - I2C_0_SDA_3V | SDA |  | Common non-critical I2C |

| OFT001 16 pin header (J3) | ESP32 HUUZZAH 16 pin header (JP1) | ESP32 silicon | Comments |
| --- | --- | --- | --- |
| 1 – INT_0 | IO21 |  | Interrupt from MPU9250 |
| 2 - I2C_1_SDA_3V | IO17 |  | Critical MPU I2C |
| 3 - I2C_1_SCL_3V | IO16 |  | Critical MPU I2C |
| 4 - N/C | MISO |  |  |
| 5 - N/C | MOSI |  |  |
| 6 - N/C | SCK |  |  |
| 7 - CALIB | A5_IO4 |  | System calibration button |
| 8 - UART_1_CLIENT_RXD | A4_IO36 |  | L8-M39 GPS UART (NMEA) |
| 9 - UART_1_CLIENT_TXD | A3_I39 |  | L8-M39 GPS UART (NMEA) |
| 10 – N/C | A2_I34 |  |  |
| 11 – N/C | A1_DAC1 |  |  |
| 12 – N/C | A0_DAC2 |  |  |
| 13 - GND | GND |  |  |
| 14 – N/C | N/C |  |  |
| 15 - +3.3V (VDD) | +3.3V |  |  |
| 16 - RESET | RESET |  | 2-Way reset |

| OFT001 Ext GPS Ant. (J1) | Comments |
| --- | --- |
| 1 – AGND | External active GPS antenna ground with VDD DC applied for power feed |
| 2 – EXTANT | External active GPS antenna signal with + |

OFT001 v1 addressing

| I2CBus | Addr | Function | Comments |
| --- | --- | --- | --- |
| 0 | 0x77 (0x761) | BME280 |  |
| 1 | 0x68 (0x692) | MPU9250 |  |

- Alternative address if NULL resistor R4 is mounted

- Alternative address if NULL resistor R7 is mounted

OFT001 v1 operating specifications

Operational conditions:

| Parameter | Nom | Max | Min |
| --- | --- | --- | --- |
| Voltage | 3.3 V | 3.6 V | 3.0 V |
| Current | 30 mA + | 120 mA + | - |
| Power dissipation | 100 mW | 400 mW | - |
| Ambient temp | - | 85 degC | -40 degC |

GPS antenna operational conditions:

The built in GPS patch antenna provides the following characteristics (probably not really true!?):

Following external GPS antenna characteristics applies:

| Parameter | Value |
| --- | --- |
| DC feed: | 3.6V – 2.5V |
| DC current: | <??? |
| Center frequency: | 1575.42MHz |
| Band width: | >5MHz |
| VSWR: | <2 (Typ.) |
| Polarization: | RHCP or Linear |
| Noise figure: | <1.5dB |
| Gain (antenna): | >-2dBi |
| Gain (embedded LNA): | 20dB (Typ.) |
| Total gain: | >18dBi(Typ.) |

OFT001 v1 schematics

Figure 8: OFT001 v1 circuit schematics

OFT001 v1 - third party data sheets

L80-M39 GPS receiver:

http://docs-emea.rs-online.com/webdocs/147d/0900766b8147dbf2.pdf

BME280 Barometric pressure sensor:

https://ae-bst.resource.bosch.com/media/_tech/media/datasheets/BST-BME280-DS002.pdf

MPU9250 9-Axis Accelerometer, Gyroscope and Magenetometer:

http://43zrtwysvxb2gf29r5o0athu-wpengine.netdna-ssl.com/wp-content/uploads/2015/02/PS-MPU-9250A-01-v1.1.pdf

TRI-color system status LED

https://docs.broadcom.com/docs/AV02-4194EN

GPS status LEDs

https://docs-emea.rs-online.com/webdocs/156b/0900766b8156b2c9.pdf

OFT001 v1 PCB implementation

KICAD 5.1.4 has been used to develop the schematics and PCB design. It is targeted for a 4 layer FR4 1.6 mm PCB with a minimum of 6 mil inch track width- and spacing, and a tolerance of 10%. Minimum via drills is 0.3 mm.

PCB Stack-up

The PCB stack-up used is the one produced by PCBway (https://www.pcbway.com/orderonline.aspx)

Figure 9: OFT001 v1 – 4-layer PCB stack-up

Front/top Signal Cupper layer

Figure 10: OFT001 v1 - Front/top Signal Cupper layer

Back/bottom Signal CU layer

Figure 11: OFT001 v1 - Back/bottom Signal CU layer

Top/front embedded GND CU layer

Figure 12: OFT001 v1 - Top/front embedded GND CU layer

Back/bottom embedded VDD/3.3V CU layer

Figure 13: OFT001 v1 - Back/bottom embedded VDD/3.3V CU layer

Front mask

Figure 14: OFT001 v1 - Front mask

Front paste

Figure 14: OFT001 v1 – Front paste

Back mask

Figure 15: OFT001 v1 – Back mask

Front silk

Figure 16: OFT001 v1 – Front silk

Back silk

Figure 17: OFT001 v1 – Back silk

Various user meta data layers

Figure 18: OFT001 v1 – Various user meta data layers

External GPS antenna Impedance matching

The external GPS antenna trace needs to be carefully impedance matched to 50 ohms in order to get the signal losses low and to avoid reflections. Based on PCBWay PCB substrate stack-up, the top cupper strip thickness (T) is estimated to be 35um (+/- 10%), the distance to next ground layer (H) is estimated to be 0.11 mm (+/- 10%), and the dielectric permittivity in between is estimated to be 4.5 F/m @ 1.5 GHz (I need to recalculate with 4.29 Nominal instead).

With the constraints of 6 mil inch strip widths/spacing we end up with a 0.2 mm (W) strip width, and a 6 mil/0.16 mm (S) horizontal spacing to earth - forming a coplanar wave guide with a ground plane, of roughly 2 mm length (L) matching the 50 Ohm antenna strip impedance.

Figure 19: OFT001 v1 – External GPS antenna layout

With this layout, it is expected that tolerances of non-impedance-controlled PCB manufacturing for this strip to stay in between 41-55.5 Ohms, and losses to be less than 10 mdB.

Since the 1.5Ghz GPS wavelength in cupper is roughly 2 dm (100 times the strip length), a few reflections due to impedance matching imperfections should not degrade the antenna signal significantly.

Figure 20: OFT001 v1 – External GPS antenna layout – top CU layer

Figure 21: OFT001 v1 – External GPS antenna – Impedance calculations

Appendix A – ESP32 Mother board reference

Figure 22: Ada fruit ESP32 Huzzah Feather board

Figure 23: Ada fruit ESP32 Huzzah Feather board schematics

Appendix B – OFT001 v1 Build of material

| ref | value5 | footprint | datasheet | part | description |
| --- | --- | --- | --- | --- | --- |
| J1 | Ext GPS antenna connector | Open_Telemetry:U.FL_Molex_MCRF_73412-0110_Vertical | http://www.molex.com/pdm_docs/sd/734120110_sd.pdf | 73412-0110 | 50 Ohms, MCRF, PCB Vertical Jack Receptacle, SMT, 1.25mm Mounted Height |
| IC1 | L80-M39 | Open_Telemetry:L80-M39 | http://docs-emea.rs-online.com/webdocs/147d/0900766b8147dbf2.pdf | L80-M39 | L80 GPS Module |
| C6 | 10uF | CAPPM3528X210N | https://api.kemet.com/component-edge/download/datasheet/T491B106K016AT7280.pdf | T491B106K016AT7280 | Tantalum Capacitors - Solid SMD 16V 10uF 1311 10% ESR=2ohms |
| C7 | 10uF | CAPPM3528X210N | https://api.kemet.com/component-edge/download/datasheet/T491B106K016AT7280.pdf | T491B106K016AT7280 | Tantalum Capacitors - Solid SMD 16V 10uF 1311 10% ESR=2ohms |
| C8 | 10uF | CAPPM3528X210N | https://api.kemet.com/component-edge/download/datasheet/T491B106K016AT7280.pdf | T491B106K016AT7280 | Tantalum Capacitors - Solid SMD 16V 10uF 1311 10% ESR=2ohms |
| C3 | 10nF | Open_Telemetry:CAPC1608X87N | https://api.kemet.com/component-edge/download/datasheet/C0603C103K4RACTU.pdf | C0603C103K4RACTU | KEMET - C0603C103K4RACTU - CAP, 0.01F, 16V, 10%, X7R, 0603 |
| R2 | 1k | Open_Telemetry:RESC0603X26N | https://www.digikey.se/en/datasheets/yageo/yageo-pyu-r_marking_2 | RC0201JR-071KL | YAGEO - RC0201JR-071KL - RES, THICK FILM, 1K, 5%, 0.5W, 0201, REEL |
| R6 | 1k | Open_Telemetry:RESC0603X26N | https://www.digikey.se/en/datasheets/yageo/yageo-pyu-r_marking_2 | RC0201JR-071KL | YAGEO - RC0201JR-071KL - RES, THICK FILM, 1K, 5%, 0.5W, 0201, REEL |
| R3 | 100k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-07100KL | YAGEO - RC0201JR-07100KL - RES, THICK FILM, 100K, 5%, 0.5W, 0201 |
| R5 | 100k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-07100KL | YAGEO - RC0201JR-07100KL - RES, THICK FILM, 100K, 5%, 0.5W, 0201 |
| R8 | 10k | RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-0710KL | THICK FILM CHIP RESISTORS |
| R9 | 10k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-0710KL | THICK FILM CHIP RESISTORS |
| R1 | 10k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-0710KL | THICK FILM CHIP RESISTORS |
| C4 | 10nF | Open_Telemetry:CAPC1608X87N | https://api.kemet.com/component-edge/download/datasheet/C0603C103K4RACTU.pdf | C0603C103K4RACTU | KEMET - C0603C103K4RACTU - CAP, 0.01F, 16V, 10%, X7R, 0603 |
| C5 | 100nF | Open_Telemetry:CAPC1005X55N | https://content.kemet.com/datasheets/KEM_C1002_X7R_SMD.pdf | C0402C104M8RACTU | Multilayer Ceramic Capacitors MLCC - SMD/SMT 10volts 0.1uF X7R 20% |
| LED1 | EXT ANT | Open_Telemetry:LEDC1608X65N | https://docs-emea.rs-online.com/webdocs/156b/0900766b8156b2c9.pdf | SML-D13DWT86A | ROHM SML-D13DWT86A, SML 608 nm Orange LED, 1608 (0603) Milky White SMD package |
| S1 | RST | Open_Telemetry:ATS2D1G | http://www.mouser.com/ds/2/60/ats-1100738.pdf | ATS2D1G_NC_LFS | Tactile Switches 1VA 32VDC 50mA On-(On) Sldr IP54 |
| S2 | CALIB | Open_Telemetry:ATS2D1G | http://www.mouser.com/ds/2/60/ats-1100738.pdf | ATS2D1G_NC_LFS | Tactile Switches 1VA 32VDC 50mA On-(On) Sldr IP54 |
| C1 | 10nF | Open_Telemetry:CAPC1608X87N | https://api.kemet.com/component-edge/download/datasheet/C0603C103K4RACTU.pdf | C0603C103K4RACTU | KEMET - C0603C103K4RACTU - CAP, 0.01F, 16V, 10%, X7R, 0603 |
| C2 | 10nF | Open_Telemetry:CAPC1608X87N | https://api.kemet.com/component-edge/download/datasheet/C0603C103K4RACTU.pdf | C0603C103K4RACTU | KEMET - C0603C103K4RACTU - CAP, 0.01F, 16V, 10%, X7R, 0603 |
| R15 | 10k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-0710KL | THICK FILM CHIP RESISTORS |
| R16 | 10k | Open_Telemetry:RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-0710KL | THICK FILM CHIP RESISTORS |
| R10 | 62 | Open_Telemetry:RESC1005X40N | http://www.yageo.com/documents/recent/PYu-RC0402_51_RoHS_L_6_r.pdf | RC0402JR-0762RL | Thick Film Resistors - SMD 62 OHM 5% |
| R11 | 62 | Open_Telemetry:RESC1005X40N | http://www.yageo.com/documents/recent/PYu-RC0402_51_RoHS_L_6_r.pdf | RC0402JR-0762RL | Thick Film Resistors - SMD 62 OHM 5% |
| R12 | 62 | Open_Telemetry:RESC1005X40N | http://www.yageo.com/documents/recent/PYu-RC0402_51_RoHS_L_6_r.pdf | RC0402JR-0762RL | Thick Film Resistors - SMD 62 OHM 5% |
| R13 | 62 | Open_Telemetry:RESC1005X40N | http://www.yageo.com/documents/recent/PYu-RC0402_51_RoHS_L_6_r.pdf | RC0402JR-0762RL | Thick Film Resistors - SMD 62 OHM 5% |
| LED2 | GPS LCK | Open_Telemetry:SMLD15YWT86 | http://www.rohm.com/web/global/datasheet/SML-D13FW/sml-d13x-e | SML-D13FWT86 | ROHM SML-D13FWT86, SML 566 nm Green LED, 1608 (0603) Milky White SMD package |
| R4 | NULL | RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-070RL | YAGEO (PHYCOMP) - RC0201JR-070RL - RES, THICK FILM, 0R, 5%, 0.05W, 0201 |
| R7 | NULL | RESC0603X26N | http://www.yageo.com/documents/recent/PYu-RC0201_51_RoHS_L_8.pdf | RC0201JR-070RL | YAGEO (PHYCOMP) - RC0201JR-070RL - RES, THICK FILM, 0R, 5%, 0.05W, 0201 |
| R14 | 130 | Open_Telemetry:RESC1005X40N | http://www.vishay.com/docs/20035/dcrcwe3.pdf | CRCW0402130RJNED | RESISTOR, 0402 130 Ohms +/-5% 0.063 W |

Appendix C – OpenFly Telemetry repositories

OFT HW Repositories:

https://github.com/jonasbjurel/open_telemetry_HW

OFT SW Repositories:

https://github.com/jonasbjurel/open_telemetry

OFT SW Platform Repositories:

https://github.com/jonasbjurel/coreNoStopRTOS

3PP dependency Repositories:

ESPRESSIF
FREERTOS
ADAFRUIT ESP32

Appendix D – OpenFly Telemetry WIKI

TBD